ScourPro

Automated Scour Surface Profiler — Product Specification Sheet — Proof-of-Concept (P1)

| Document ID | SCOURPRO-SPEC-P1-v0.1 |
| --- | --- |
| Status | DRAFT — duplicate this file per revision |
| Prepared by | TBD |
| Reviewed / Approved by | TBD |
| Date | TBD |
| Applicable prototype | P1 (Proof-of-Concept) |
| Target users | Indian hydraulics research & teaching labs (IITs, NITs, CWPRS) |

How to use: fill Target values at design freeze; fill Measured values only from documented tests (link test report ID in Notes). Any cell left TBD blocks release sign-off. Duplicate the file and bump the version for each iteration; log changes in §12.

1. Product Overview

| Product name / SKU | ScourPro |
| --- | --- |
| Target users | Indian hydraulics research & teaching labs (IITs, NITs, CWPRS) |
| Primary use case | Automated scour bed profiling after drained flume tests |
| Configuration | 2-axis Cartesian gantry: GT2 belt-driven X (width) and Y (length); sensor fixed (no Z motion axis) |
| Measurement principle | Single-point non-contact depth reading at each X–Y grid point; sensor moved over the bed, ToF reading gives depth |
| Mounting | Bolt-down to a fixed rail on the flume side walls |
| Target flume | 0.5 m (L) × 0.3 m (W) × 0.8 m (H); PoC fixed to 0.3 m width |
| Not-to-exceed build cost | ₹60,000 (PoC ceiling) |

2. Measurement & Motion Performance

Accuracy verified after multi-point calibration against a stepped datum block, checked on a sand-textured (matte) reference surface. Report test ID in Notes.

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| Motion axes | 2 | 2 (X, Y) | TBD | Belt X + belt Y; sensor fixed |
| Depth accuracy | ±5 mm | ±1 mm | TBD | Multi-point calibration; ISO-style repeat reads |
| Depth resolution / precision | 1 mm | ≤0.5 mm | TBD | Repeat reads, static target |
| XY positioning accuracy | ±1 mm | ±0.5 mm | TBD | Commanded vs. measured, dial gauge |
| X travel (width) | 300 mm | 300 mm | TBD | Full flume cross-section |
| Y travel (length) | 500 mm | 500 mm | TBD | Along flume length |
| Grid spacing (user-set) | 10 mm | 10–50 mm | TBD | Configurable per run |
| Z measurement window | TBD | TBD | TBD | Must cover full scour depth (~150 mm) from fixed height |
| Full-grid scan time | ≤20 min | ≤10 min | TBD | Representative grid, full area |

2.1  Work Envelope

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| X-axis travel (width) | 300 mm | 300 mm | TBD | Hard stop + soft limit values |
| Y-axis travel (length) | 500 mm | 500 mm | TBD | Hard stop + soft limit values |
| Sensor standoff height | TBD | TBD | TBD | Fixed; set to cover full depth range |
| Flume width compatibility | 0.3 m | 0.3 m | TBD | PoC fixed; adjustable is a later item |

3. Actuation & Transmission

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| Motor type (X, Y) | TBD | TBD | TBD | Stepper; NEMA 17 class expected |
| Driver | TBD | TBD | TBD | A4988 / TMC2209 class; driven by Pico |
| Transmission (X) | GT2 6 mm belt | GT2 6 mm belt | TBD | Open-ended, anchored both ends |
| Transmission (Y) | GT2 6 mm belt | GT2 6 mm belt | TBD | Dual-side drive to resist racking |
| Homing / limit switches | Required (both axes) | Required (both axes) | TBD | Home every run for repeatable origin |
| Power-off position hold | TBD | TBD | TBD | Belt is not self-locking; evaluate need |

4. Materials & Structure

| General structure | V-slot aluminium extrusion (frame doubles as linear rail) |
| --- | --- |
| Brackets & mounts | 3D-printed (PETG/ASA recommended; document print params) |
| Linear motion | V-wheels on extrusion; eccentric spacers for preload |
| Moisture / corrosion | Splash-resistant only (bed is drained before use) |
| Prohibited | PLA in load-bearing structural paths (creep) |

Acceptance criterion: positioning and depth-accuracy specs in §2 govern structural adequacy — not material choice per se.

5. Electrical & Power

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| Input supply | 12 V DC | 24 V DC | TBD | Single PSU; state final value at build |
| Logic supply | 5 V | 5 V | TBD | Buck converter from main rail; do not power Pico off motor rail |
| Controller | RPi Pico | RPi Pico | TBD | Drives steppers via drivers |
| External interface | USB | USB (PoC) → Wi-Fi (final) | TBD | USB to laptop for PoC; Wi-Fi is a later item |
| Driver decoupling | Required | Required | TBD | Bulk capacitor across each driver motor-supply |
| Inline fuse | Required | Required | TBD | On main supply |

6. Control & Software

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| Firmware | MicroPython | MicroPython | TBD | On the Pico |
| Operation | Grid + spacing entry, start | Same, refined UI | TBD | Automated raster scan |
| Output format | CSV (x, y, z) | CSV + plot + heatmap | TBD | CSV for PoC; visuals in later versions |
| 3D surface generation | Off-device (post) | Off-device (post) | TBD | Python / ParaView from CSV |
| Calibration | Multi-point datum block | Guided routine | TBD | Stepped block, matte/sand surface |

7. Environment, Safety & Compliance

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| Operating temperature | 10 °C | 10–80 °C | TBD | Indoor lab; per user spec |
| Ingress protection | None (PoC) | None (PoC) | TBD | Basic enclosure; no formal IP rating for PoC |
| Compliance | None (research PoC) | None (research PoC) | TBD | Research prototype; not a sold product |

8. Mechanical & Interfaces

| Flume attachment | Rail with leveling feet resting on / bolted to both side walls |
| --- | --- |
| Leveling | Adjustable feet; rail plane defines the Z datum for all depth readings |
| Sensor interface | Fixed sensor housing on the trolley (no moving Z axis) |
| Cable routing | Drag chain along the beam; keep cables off the moving bed |

9. Reliability, Serviceability & Lifecycle

| Parameter | Minimum | Target | Measured | Test method / Notes |
| --- | --- | --- | --- | --- |
| Duty rating | Occasional lab use | Occasional lab use | TBD | Not continuous / production duty |
| Belt service interval | TBD | TBD | TBD | Inspect tension periodically |
| Spares | TBD | TBD | TBD | Printable bracket STLs published |

10. Acceptance Test Matrix

Every Measured cell above must trace to a row here. One report per test, filed by ID.

| Test ID | Test | Method / Standard | Pass criterion |
| --- | --- | --- | --- |
| T-01 | Depth accuracy | Multi-point datum block, matte surface, repeat reads | Within ±1 mm of target (§2) |
| T-02 | XY positioning accuracy | Commanded vs. measured, dial gauge | Within ±0.5 mm target (§2) |
| T-03 | Full-grid scan time | Representative grid over full area | ≤ target (§2) |
| T-04 | Homing repeatability | Repeated home + return to reference point | Origin repeatable within XY spec |
| T-05 | Calibration routine | Run datum-block calibration, verify fit | Residuals within depth accuracy spec |
| T-06 | Belt racking (Y) | Full Y travel, check beam squareness | No skew / bind across travel |
| T-07 | End-to-end scan | Full scan of a real scoured bed, export CSV | Complete grid, no missed points |

11. Open Engineering Items

- Sensor selection on sand — confirm the chosen ToF/laser sensor holds ±1 mm on matte, possibly damp sand across the full depth range (bench test before design freeze).

- Z measurement window — fix the sensor standoff height so the depth window covers the full ~150 mm scour depth without moving; this drives the sensor spec.

- No-Z fallback — with no motion Z axis there is no contact-probe backup; decide whether a fallback is needed if the ToF underperforms.

- Belt racking on the 500 mm Y axis — single vs. dual-side drive, or a stiff enough beam; confirm before build.

- Rail leveling / datum — rail flatness along Y directly biases depth; decide leveling method vs. a per-point correction map.

- Power-off hold — belts don't self-lock; determine whether holding torque or a brake is required.

12. Revision Log

| Rev | Date | Author | Changes |
| --- | --- | --- | --- |
| v0.1 | TBD | TBD | Initial PoC draft |